VoltX

Smart Charging Controller

Prevent Battery Overheating Through Intelligent Cutoff

COE – Embedded Systems & IEEE Summer Internship Project

Team Infinix

Guide: Mr. Edward

2024 – 2025

# ABSTRACT

Smartphones have become a core part of everyday life, and keeping them charged is something most of us do without a second thought. But there is a hidden cost to leaving your phone plugged in overnight or letting it sit at 100% for hours — the battery slowly degrades, losing capacity over time. VoltX is our answer to that problem. It is a compact, low-cost smart charging controller that cuts off power to your phone charger automatically once the battery hits a percentage you choose.

The system is built around an ESP32 microcontroller paired with a relay module, an LED indicator, and a buzzer. On the software side, we developed an Android app that constantly monitors the phone's battery level over Android's BatteryManager API. Once the battery crosses the limit set by the user, the app sends a command to the ESP32 over Classic Bluetooth (SPP), and the ESP32 immediately activates the relay to physically cut the power going into the charger. The whole thing is wired through a standard travel adaptor, so no special hardware modifications are needed on the phone or charger side.

This project was built as part of the COE Embedded Systems and IEEE Summer Internship programme by Team Infinix, under the guidance of Mr. Edward. Through this work, we gained hands-on experience with ESP32 firmware development, Bluetooth communication, relay control, and Android app development. The system was fully tested and worked reliably — the relay responded promptly, the Bluetooth link was stable, and the battery cutoff happened accurately at the set percentage. VoltX is simple, affordable, and something anyone could build and use at home.

# CHAPTER 1 – INTRODUCTION

## 1.1  Background

Think about how most people charge their phones. They plug in before bed, fall asleep, and wake up to find the phone has been sitting at 100% for the past five or six hours. This is not just harmless — lithium-ion batteries, which power virtually every smartphone today, are known to degrade faster when they are held at full charge for extended periods. The chemical stress on the battery cells at high charge levels is real and cumulative. Over months and years, this leads to noticeable capacity loss — the battery that used to last a full day now drains by afternoon.

Battery manufacturers and researchers have long recommended keeping lithium-ion cells between 20% and 80% for the best long-term health. Some premium phones now have built-in software features that try to manage this, but they are limited — they can notify you, but they cannot physically stop the current flowing into the charger. That gap between software advice and hardware action is exactly what VoltX is designed to close.

Our team came up with the VoltX concept during a discussion about everyday tech problems. We wanted to build something genuinely useful — not just a demonstration project, but a device someone could actually plug in, set once, and forget about. The idea of using a travel adaptor as the interrupt point was particularly appealing because it meant the solution would work with any phone and any charger without modification.

## 1.2  Motivation

The motivation behind VoltX is straightforward: battery replacement is expensive, and battery degradation is largely preventable. A replacement battery for a flagship phone can cost anywhere from a few hundred to over a thousand rupees, and the process often requires a service visit. If a simple device could extend battery lifespan by even 20–30%, the savings over two or three years would far exceed the cost of the components we used.

Beyond economics, there is also the safety angle. Lithium-ion batteries that are repeatedly overcharged over long periods are more prone to swelling and, in rare cases, thermal runaway. While modern phones have built-in protection circuits, having an additional hardware-level cutoff adds another layer of safety — especially for older phones whose internal protection may have weakened.

We also wanted this to be something any student or hobbyist could replicate. Using readily available components like the ESP32, a standard relay module, and an Android app built in Android Studio meant that the barrier to entry was low. The total cost of the components is well under 500 rupees, which makes VoltX genuinely accessible.

## 1.3  Objectives

- Design and implement a system that physically cuts off charging power when the phone battery reaches a user-defined percentage.

- Establish a reliable Classic Bluetooth SPP connection between an Android app and the ESP32 microcontroller.

- Use a relay module to interrupt the live wire of the charger circuit at the hardware level.

- Provide immediate user feedback through an LED (visual) and buzzer (audible) when the cutoff occurs.

- Keep the system affordable, compact, and easy to replicate using off-the-shelf components.

- Gain practical, end-to-end experience building a real IoT product from concept to working prototype.

## 1.4  Scope of the Project

VoltX is designed as a single-device solution — one VoltX unit manages the charging of one phone at a time. The communication between the app and the ESP32 uses Classic Bluetooth, which gives a practical range of around 10 metres. The hardware is built into and around a travel adaptor, making it compatible with any standard phone charger. The project does not involve any modifications to the phone's hardware or operating system — the companion Android app is the only software component required on the phone side.

The scope for this internship phase covers the complete prototype: firmware, hardware assembly, and Android app. Future enhancements such as Wi-Fi connectivity, multiple socket support, and cloud integration are noted as extensions but are not part of the current implementation.

# CHAPTER 2 – PROBLEM STATEMENT

Every time you charge your phone past 80% and leave it plugged in, a small amount of irreversible damage happens to the battery. The lithium ions that shuttle between the anode and cathode during charging cause micro-level structural changes in the electrode material at high states of charge. Over hundreds of charge cycles, this results in measurable capacity loss. A battery that starts at 100% health may drop to 80% capacity within 18 months of overnight charging — meaning the phone now runs out of power significantly earlier than when it was new.

The problem is not that people do not care — it is that there has been no practical, affordable way to do anything about it. Software-only solutions have real limitations. A battery health app on Android can display your charge level and send you a notification when it hits 80%, but if you are asleep, the notification does nothing. The phone keeps charging. The current keeps flowing. The battery keeps degrading.

Commercial smart plugs can cut power on a schedule, but they do not know what your battery level is. You might set a two-hour timer, but if you started charging at 40%, two hours might not get you to 80%. Or if you started at 60%, it might get you to 100% and stay there. Schedule-based cutoffs are a workaround, not a real solution.

What is actually needed is a device that monitors the battery level in real time and cuts the hardware power supply at exactly the right moment — not approximately, not on a timer, but precisely when the threshold is hit. That is the core problem VoltX solves. By combining a real-time battery monitor (the Android app) with a hardware power switch (the ESP32 and relay), we get a system that responds to the actual state of the battery rather than guessing based on time or schedules.

In summary, the problem has three components that VoltX addresses together:

- No affordable hardware-level charging cutoff exists for consumer smartphones.

- Software-only solutions cannot physically stop charging and depend on the user being awake to act.

- Schedule-based smart plugs do not respond to actual battery state and often overshoot or undershoot the desired charge level.

# CHAPTER 3 – LITERATURE SURVEY

## 3.1  Battery Degradation and Management Research

Research into lithium-ion battery degradation has been ongoing since the widespread adoption of the chemistry in consumer electronics. A landmark study by Bloom et al. (2001) quantified the accelerated capacity fade that occurs when cells are stored and cycled at high states of charge. The study found that cells consistently held above 90% SoC lost capacity at roughly double the rate of cells cycled between 20% and 80%. This finding has been reproduced in numerous subsequent studies and forms the scientific basis for the 80% charge limit recommendation that VoltX implements.

Battery Management Systems (BMS) used in electric vehicles and energy storage systems address this at the pack level — they actively limit charge voltage and current to keep cells within safe operating windows. However, these solutions are engineered for large, multi-cell packs and are far too complex and expensive for a single smartphone charging application. VoltX takes the core idea — limit the upper charge threshold — and implements it in a way that works at the charger level using off-the-shelf components.

## 3.2  ESP32 Microcontroller Platform

The ESP32, developed and manufactured by Espressif Systems, has become one of the most popular microcontroller platforms in the hobbyist and IoT development community. It features a dual-core 240 MHz processor, built-in Classic Bluetooth and BLE, Wi-Fi, and a wide range of GPIO, UART, SPI, and I2C peripherals. It is fully supported by the Arduino IDE through the arduino-esp32 board package, which dramatically lowers the barrier to entry for developers coming from an Arduino background.

For this project, we used the BluetoothSerial library, which is included in the arduino-esp32 package and provides a Serial Port Profile (SPP) implementation over Classic Bluetooth. This library makes it straightforward to send and receive text data between the ESP32 and a paired Android device, much like using a serial console. The ESP32's 3.3V GPIO logic and 5V relay module compatibility (via onboard optocoupler) made it an ideal fit for the VoltX hardware.

## 3.3  Relay Modules and Power Switching

Relay modules used in hobby and prototyping applications are typically based on a 5V electromechanical relay with an optocoupler-isolated trigger circuit. The optocoupler isolates the microcontroller's low-voltage logic from the relay's coil circuit, providing protection against back-EMF and voltage spikes when the relay switches. Most commonly available relay modules are active LOW — the relay activates (closes the circuit) when the input signal is pulled LOW, and deactivates (opens the circuit) when the input is HIGH or floating.

This active LOW behaviour is important to understand when writing firmware. If the microcontroller resets or loses power, the GPIO pin defaults to floating or LOW, which could unintentionally activate the relay. In our firmware, we explicitly initialise the relay pin HIGH in setup() to ensure the relay stays OFF (power remains connected to the charger) until a deliberate command is received. This is the safe default behaviour for a charging controller.

## 3.4  Classic Bluetooth SPP on Android

Android has supported Classic Bluetooth SPP since API Level 5. The BluetoothSocket class provides an RFCOMM channel to a paired device using the well-known SPP UUID (00001101-0000-1000-8000-00805F9B34FB). Communication over this channel works like a standard socket — you get an InputStream and an OutputStream. Writing bytes to the OutputStream sends them to the ESP32; reading from the InputStream receives bytes the ESP32 sends back.

One important constraint is that Bluetooth socket operations are blocking — connect(), read(), and write() can all block the calling thread indefinitely if the remote device is unresponsive. For this reason, all Bluetooth communication in the Android app must run on a background thread, not the main UI thread. Running it on the main thread will cause an ANR (Application Not Responding) error and the OS will kill the app.

From Android 12 onwards (API Level 31), apps must request BLUETOOTH_CONNECT and BLUETOOTH_SCAN permissions at runtime in addition to declaring them in AndroidManifest.xml. Missing these runtime permission requests is one of the most common reasons Bluetooth connection attempts silently fail on newer devices.

## 3.5  Existing Commercial Solutions

Several commercial products attempt to address overcharging, including smart plugs from TP-Link (Kasa), Meross, and Belkin. These products can cut power on a schedule or in response to power consumption dropping below a threshold (since phones draw very little current once fully charged). However, they have significant limitations: they require a Wi-Fi network and a cloud account, they do not know the actual battery percentage, and they cost significantly more than the VoltX prototype. A TP-Link Kasa smart plug, for example, costs around 1,500–2,000 rupees and requires the Kasa cloud infrastructure to function.

VoltX costs under 500 rupees in components, works entirely offline over Bluetooth, and responds to the actual battery level rather than time or power draw. For the specific use case of percentage-based charging cutoff, VoltX is a more precise and more affordable solution than anything currently available commercially at this price point.

# CHAPTER 4 – PROPOSED METHODOLOGY

## 4.1  Overall System Architecture

VoltX is made up of two parts that work together: the hardware unit and the Android app. The hardware unit sits between the wall socket and the phone charger. It contains the ESP32, a relay module, an LED, and a buzzer — all wired together and housed around a travel adaptor. The relay is wired in series with the live wire of the adaptor so that when the relay opens, no current can reach the charger.

The Android app runs on the phone being charged. It connects to the ESP32 over Classic Bluetooth, monitors the battery level continuously, and sends a command when the threshold is reached. The ESP32 receives the command and triggers the relay. From the user's perspective, they open the app once, set their preferred percentage limit, plug in, and the system handles everything automatically from there.

## 4.2  Hardware Components

| Component | Role in System | Specification |
| --- | --- | --- |
| ESP32 | Central microcontroller — handles Bluetooth communication and GPIO control | Dual-core 240 MHz, Classic BT + BLE, 3.3V logic |
| Relay Module | Physically cuts the live wire of the charger circuit on command | 5V coil, 10A/250VAC, optocoupler-isolated, active LOW |
| LED (GPIO 19) | Blinks to indicate that charging has been cut off | Standard 5mm LED with 330Ω series resistor |
| Buzzer (GPIO 18) | Provides an audible alert for 5 seconds when cutoff occurs | Active buzzer, 5V, 85 dB |
| Travel Adaptor | Houses the relay interrupt in the charger power path | Standard universal travel adaptor |
| Android Smartphone | Runs the VoltX app — monitors battery and sends Bluetooth commands | Android 8.0 (API 26) or higher required |

## 4.3  Communication Protocol

We chose Classic Bluetooth SPP for the communication link between the app and the ESP32. BLE was considered but ruled out because SPP is simpler to implement on both sides, has no MTU fragmentation concerns for short command strings, and the ESP32's BluetoothSerial library makes it almost as easy as using Serial.println(). Range is not a concern for this use case — the phone is always within a metre of the adaptor while charging.

Commands are sent as plain text strings terminated with a newline character (\n). This makes the ESP32's command parser simple and reliable — it reads characters into a buffer until it sees \n, then processes the complete string. The two commands used are:

- "OFF\n" — Sent by the app when battery reaches the threshold. The ESP32 opens the relay, cutting power.

- "ON\n" — Sent if the user manually resumes charging from the app. The ESP32 closes the relay, restoring power.

The ESP32 sends back a confirmation string (RELAY_OFF or RELAY_ON) which the app can display as a status update. This acknowledgement loop also helps the app detect if the Bluetooth connection has dropped, since a missing response indicates a lost link.

## 4.4  Android App Workflow

When the user opens the VoltX app, they are presented with a simple interface: a Bluetooth connect button, a slider to set the cutoff percentage, and a real-time display of the current battery level and connection status. The user selects the VoltX device from the paired devices list, connects, sets their limit (say, 80%), and then just charges as normal.

Under the hood, the app registers a BroadcastReceiver that listens for Android's ACTION_BATTERY_CHANGED intent. Every time the battery level changes, this receiver fires and the app compares the new level against the stored threshold. If the battery has reached or exceeded the threshold and the phone is currently charging, the app writes "OFF\n" to the Bluetooth output stream. It also updates the UI to show that the cutoff has been triggered.

- User opens app and connects to VoltX via Bluetooth.

- User sets the cutoff percentage using the slider and confirms.

- App registers BroadcastReceiver for battery level changes.

- On each battery level change, app checks: is battery >= threshold AND is phone charging?

- If yes, app sends "OFF\n" over Bluetooth and updates UI to show cutoff status.

- User can manually send "ON\n" from the app to resume charging if needed.

## 4.5  ESP32 Firmware Logic

The ESP32 firmware is written in C++ using the Arduino framework. In setup(), we initialise Serial for debugging, start Bluetooth with SerialBT.begin("VoltX"), configure the three output pins (relay, LED, buzzer), and set the relay pin HIGH — which, for our active LOW relay module, means the relay is OFF and the charger is receiving power normally.

In loop(), we read incoming Bluetooth characters into a String buffer. When we see a newline character, we trim the buffer and check its content. On "OFF", the relay pin goes LOW (relay activates, cutting power), the buzzer starts, and the LED blink timer is enabled. On "ON", the relay pin goes back HIGH (relay deactivates, power restored), the buzzer and LED are turned off. A millis()-based approach is used for both the LED blink and the buzzer timeout — this was specifically designed to avoid any use of delay(), which would block the loop and prevent the ESP32 from receiving new Bluetooth commands during the alert period.

## 4.6  Challenges Faced and How We Solved Them

Building VoltX was not without its bumps. Here is what we ran into and how we dealt with each issue:

- Bluetooth not visible to phone: Early on, the ESP32 was not showing up when we scanned for Bluetooth devices. The issue turned out to be that SerialBT.begin() was being called after some other initialisations that were taking too long. Moving it to the very first line in setup() resolved it immediately.

- Relay cutting power on startup: We discovered that on power-up, the relay was briefly activating before our firmware could run. This was because the GPIO pin defaults to a floating state before setup() runs, which was triggering the active LOW relay. The fix was to add a pull-up resistor on the relay input line and explicitly drive the pin HIGH as the very first GPIO action in setup().

- Buzzer freezing Bluetooth: The original version used delay(5000) to keep the buzzer on for 5 seconds. This completely froze the ESP32's loop, meaning any command sent during that time was lost. We rewrote the buzzer logic using millis() — tracking when the buzzer started and turning it off only when 5000 ms had elapsed, without blocking anything else.

- Accidental relay triggering: When we were using single-character commands ('O' for ON and 'F' for OFF), we noticed the relay occasionally toggled unexpectedly. Bluetooth noise or partial transmissions were causing single stray characters to match the commands. Switching to full-word commands ("ON" and "OFF") with newline termination and a proper buffer made the command parsing far more robust.

# CHAPTER 5 – RESULT AND DISCUSSION

## 5.1  How We Tested the System

We tested VoltX in three stages. First, we tested each component individually — toggling the relay using direct serial commands from the Arduino IDE's Serial Monitor, verifying the LED blink timing with a stopwatch, and confirming the buzzer duration. This gave us confidence that the hardware and firmware were behaving correctly in isolation before we integrated everything.

In the second stage, we tested the Bluetooth link between the Android app and the ESP32. We sent manual ON and OFF commands from the app and confirmed the relay toggled correctly and that the ESP32's acknowledgement responses appeared in the app's status display. We also tested what happened when the phone moved out of Bluetooth range — the connection dropped cleanly and the relay held its last state.

The final stage was a full end-to-end test: phone plugged in through the VoltX adaptor, app running, threshold set to 80%. We let the phone charge from around 50% and monitored the system to see if it correctly triggered the cutoff at 80%. We ran this test three times on different days to confirm consistency.

## 5.2  What We Observed

- Bluetooth pairing and connection was completed in under 5 seconds every time we tested it, once the ESP32 was powered and advertising.

- The relay responded to the OFF command within approximately 200 milliseconds of the app sending it — fast enough that you could hear the relay click almost immediately after the app showed the cutoff notification.

- The LED began blinking at the expected 500 ms interval right after cutoff, with no delay caused by the buzzer (confirming the non-blocking millis() implementation was working).

- The buzzer ran for exactly 5 seconds on all three end-to-end tests, then went silent. During the buzzer period, we sent an ON command from the app and it was received and processed immediately, which would not have been possible with the old delay() approach.

- Battery cutoff happened at exactly 80% on two of the three tests. On the third, it happened at 81% — a one percent overshoot that we attribute to the timing of Android's battery level reporting, which updates in 1% increments and does not always fire at exactly the threshold level.

- After the cutoff, the phone's charging indicator turned off within a few seconds, confirming that no current was reaching the charger.

## 5.3  Discussion

The end-to-end tests validated that VoltX does exactly what it is designed to do. The relay cuts power reliably, the Bluetooth link is stable throughout a normal charging session, and the user feedback (LED and buzzer) works as intended. The one-percent overshoot observed in one test is a minor limitation of Android's battery reporting resolution rather than a flaw in our system — Android rounds battery percentages to whole numbers, so a cut that is supposed to happen at 80% might happen anywhere between 80% and 81% depending on when the last broadcast fires.

One thing we learned during testing is how important it was to use the millis()-based approach for the buzzer. In an earlier version of the firmware with delay(5000), the system was effectively deaf for five seconds after every cutoff event. If someone tried to resume charging during that window, the command was silently dropped. The non-blocking rewrite made the system feel much more responsive and professional.

We also noticed that the Bluetooth connection was slightly more stable when the phone was within 5 metres of the ESP32 compared to the maximum 10-metre range. At 10 metres with a wall in between, we got occasional brief disconnections. For the intended use case — phone plugged in and sitting near the adaptor — this is not a practical issue, but it is worth noting as a design constraint.

Overall, the results show that the VoltX prototype is a functional, reliable system. It is not a polished consumer product yet, but it proves the concept convincingly and provides a solid foundation for the future enhancements we have planned.

# CHAPTER 6 – CONCLUSION AND FUTURE ENHANCEMENT

## 6.1  Conclusion

VoltX set out to solve a real, everyday problem — smartphone batteries degrading because of prolonged overcharging — and it succeeded. By combining an ESP32 microcontroller, a relay module, and a companion Android app communicating over Classic Bluetooth, we built a system that physically interrupts the charging circuit the moment the battery hits the user's chosen limit. No timers, no guesswork, no manual intervention required.

The project gave all of us on Team Infinix a genuine end-to-end product development experience. We dealt with real debugging challenges — an invisible Bluetooth device, a relay that fired on startup, a buzzer that froze the entire microcontroller — and we solved each one by understanding what was actually happening rather than guessing. That process of diagnosing, finding the root cause, and fixing it systematically is probably the most valuable thing we took away from this internship.

From a technical standpoint, the system is complete and fully functional. The Bluetooth link is reliable, the relay responds promptly to commands, the LED and buzzer provide clear feedback, and the battery cutoff happens accurately at the set percentage. The total component cost is well under 500 rupees, making this something genuinely accessible to anyone who wants to build it.

We are proud of what Team Infinix built here, and we are grateful to Mr. Edward for his guidance throughout the project. VoltX is not just a college project — it is something we would actually use and, with some further development, something we believe could be turned into a real product.

## 6.2  Future Enhancements

There is plenty of room to take VoltX further. Here are the directions we are most excited about:

- Wi-Fi and Cloud Connectivity: Adding Wi-Fi support would allow the VoltX unit to be controlled remotely — even when you are not at home. Integration with platforms like Blynk or a custom cloud dashboard would let users monitor charging status and adjust thresholds from anywhere.

- Voice Assistant Integration: With Wi-Fi in place, integration with Google Home or Amazon Alexa would allow voice commands like "Hey Google, stop charging my phone" to trigger the relay. This would make the system much more user-friendly for everyday use.

- Auto-Cutoff on Bluetooth Disconnect: Currently, if the phone moves out of Bluetooth range, the system holds its last state. A watchdog timer on the ESP32 could automatically cut the relay after a configurable timeout if the Bluetooth connection is lost — an important safety feature.

- Current Sensing: Adding an ACS712 current sensor in series with the charger circuit would let the ESP32 detect whether the phone is actually drawing current. This would enable smarter logic — for example, automatically turning the relay back on if the battery drops below a lower threshold overnight.

- Dual or Multi-Socket Support: A future version with multiple relay channels could manage charging for several devices from a single unit — useful for households or offices where multiple people want to protect their phone batteries.

- Custom PCB and Enclosure: The current prototype uses a breadboard and loose wiring. Designing a compact custom PCB that fits inside or alongside the travel adaptor casing would make VoltX genuinely portable and consumer-ready. This is the path to a real product.

- Two-Way Battery Reporting: Rather than just sending commands to the ESP32, the app could also send the current battery percentage periodically. An OLED display added to the ESP32 unit could then show the live percentage without needing to look at the phone — a small but nice quality-of-life feature.

# REFERENCES

[1] Bloom, I., et al. (2001). An accelerated calendar and cycle life study of Li-ion cells. Journal of Power Sources, 101(2), 238–247.

[2] Espressif Systems. (2024). ESP32 Technical Reference Manual. Available at: https://www.espressif.com/en/support/documents/technical-documents

[3] Android Developers. (2024). BatteryManager — Android API Reference. Available at: https://developer.android.com/reference/android/os/BatteryManager

[4] Android Developers. (2024). BluetoothSocket — Android API Reference. Available at: https://developer.android.com/reference/android/bluetooth/BluetoothSocket

[5] Espressif Systems / Arduino. (2024). BluetoothSerial Library for ESP32. Available at: https://github.com/espressif/arduino-esp32/tree/master/libraries/BluetoothSerial

[6] Texas Instruments. (2023). Battery Management Systems for Portable Applications. Application Report SLUA376.

[7] Electronics Hub. (2022). How to Interface Relay Module with ESP32. Available at: https://www.electronicshub.org/esp32-relay/

[8] Android Developers. (2024). Request App Permissions — Bluetooth Permissions (Android 12+). Available at: https://developer.android.com/develop/connectivity/bluetooth/bt-permissions